Looper Viewer

User Manual

Document Table of Contents

1 Background & Introduction	3

1.1 Background	3

1.2 Introduction	3

2 Software Overview	3

2.1 Main Interface	3

2.2 Menu Bar	4

2.2.1 File Functions	4

2.2.2 View Functions	4

2.2.2.1 Log Information Window	5

2.2.3 Tools Functions	5

2.2.3.1 Viewer Settings Interface	6

2.2.3.1.1 Recording & Saving Interface	6

2.2.3.1.2 Performance interface	7

2.3 Image Preview Area Overview	8

2.4 Sensor Info Preview Area	9

2.5 VIO Test Preview Area	9

2.6 Control Panel Overview	10

2.7 IMU Data Line Chart	11

3 Download & Installation	12

3.1 Download	12

3.2 Installation	12

3.2.1 Windows	12

3.2.2 Linux	12

- Background & Introduction

  - Background

- Looper Viewer is a simple graphical application developed by Looper based on the Looper Insight 9 camera. It is designed to help developers quickly use and test Looper UVC camera mode.

  - Introduction

- Looper Viewer provides video stream preview, real‑time VIO data visualization, post‑processing filters, and some debugging functions for the Looper Insight 9 camera via the InsightSDK. This document will explore the various features of the Looper camera, provide detailed explanations and operational demonstrations, and help users efficiently master the powerful capabilities of the Looper camera.

- Software Overview

  - Main Interface

The main interface of Looper Viewer is divided into six areas:

- **Area 1:** Menu bar

- **Area 2:** Image preview area

- **Area 3:** Control and processing area

- **Area 4:** VIO test preview area

- **Area 5:** Camera processing control area

- **Area 6:** IMU data line chart

Figure 2-1 Main interface layout

**Table 2-1 Main interface functional descriptions**

| Area | Name | Function Description |
| --- | --- | --- |
| Area 1 | Menu bar | Provides shortcuts to common functions |
| Area 2 | Image preview area | Displays preview of data streams |
| Area 3 | Sensor info preview area | Displays sensor information |
| Area 4 | VIO test preview area | Renders camera VIO status in 3D |
| Area 5 | Camera processing control | Image processing and control page |
| Area 6 | IMU data line chart | Real-time line chart of IMU acceleration and gyroscope |

  - Menu Bar

The menu bar is located at the top of the main interface. Buttons from left to right: File,View,Tools. They provide viewer control functions.

Figure 2-2 Menu bar

    - File Functions

Currently only the Quit function is available – click to exit the application.

Figure 2-3 File functions

    - View Functions

It is possible to control the display / hiding of each window and area within the region. By clicking the corresponding window option, you can switch between the display and hiding of the respective window.

Figure 2-4 View functions

      - Log Information Window

- Displays background log information for users to view runtime logs. There are **Copy** and **Clear** buttons in the upper‑left corner for debugging convenience.

Figure 2-5 Log information area

    - Tools Functions

Currently only the function to open the Looper Viewer settings interface – click to pop up the Viewer settings.

Figure 2-6 Tools functions

      - Viewer Settings Interface

The Viewer settings can be configured, including settings for Recording & Saving and Performance.

Figure 2-7 Viewer settings interface

        - Recording & Saving Interface

This interface provides the function to save the received data and information streams from the camera.

- Save Test Info Button: After clicking, a log file will be created in the current "./" path, and the button will change to the prompt: "Stop Saving Test Info". Then, the received data within the previous second and the last frame received data will be output and saved at a frequency of once per second until the next "Stop" click.

- Save Depth Image Button: After clicking, the received depth image will be saved to the "./saved_images/depth_" folder as a PGM file, and the saving will be paused until the "Stop" click.

- Save Gray Image Button: After clicking, the received depth image will be saved to the "./saved_images/gray_" folder as a PGM file, and the saving will be paused until the "Stop" click.

- Save RGB Image Button: After clicking, the received depth image will be saved to the "./saved_images/rgb_" folder as a jpg file, and the saving will be paused until the "Stop" click.

Figure 2-8 Recording & Saving Interface

        - Performance interface

It offers simple adjustment functions for Viewer performance.

- VSync: Allows setting the vertical synchronization option for the entire interface.

- UI Font Scale: Adjusts the font size of the entire interface by dragging the slider. Currently, it is limited to 80% to 150%.

Figure 2-9 Performance interface

  - 2.3 Image Preview Area Overview

Located on the left side of the Looper Viewer main interface, this is the core display area for showing image data read by the application.

When the Viewer cannot detect a Looper device, it shows prompts like "Camera no connection", and automatically matches and displays images in the corresponding windows once a Looper device is connected.

Figure 2-10 Image preview area

  - 2.4 Sensor Info Preview Area

Located at the bottom of the Looper Viewer main interface, it displays received sensor information in real time, including current reception frequency and the frequency displayed on the screen.

Figure 2-11 Sensor Information Preview Area

  - 2.5 VIO Test Preview Area

Provides real-time 3D rendering of VIO motion information, allowing developers to verify VIO data accuracy. It offers features such as 3D trajectory display and camera orientation. You can hide/show the trajectory, axes, and reference plane via the options, and reset the trajectory with the Reset button.

Figure 2-12 VIO test preview area

  - 2.6 Control Panel Overview

Located on the right side of the Looper Viewer main interface, this is the area for camera control. After clicking and dragging configuration options, click the Apply button below to send the configuration to the Looper and modify the camera settings; the processed image and VIO data are then returned. Click Reset to restore default Looper configuration. Close closes the Camera Setting window.

Figure 2-13 Control panel

  - 2.7 IMU Data Line Chart

Displays received IMU information more intuitively via a line chart, facilitating developers to verify IMU data accuracy. Switch between Acceleration and Gyroscope to view different IMU data. Toggle Auto-scroll to pause scrolling. Use the mouse wheel to zoom in/out for detailed data inspection.

Figure 2-14 IMU data line chart

- Download & Installation

  - Download

You can download Looper Viewer from the official Looper repository:

GitHub：https://github.com/LooperRobotics/insight-view

Official Looper SDK repository:

GitHub：

https://github.com/LooperRobotics/insight-sdk](https://github.com/LooperRobotics/insight-sdk

  - Installation

    - Windows

After downloading, simply extract the package to the desired directory – no additional installation is required. The folder contains the `Insight-Viewer.exe` executable and the default configuration file `configs/default.json`.

Figure 3-1 Windows

    - Linux

After downloading, the first time you run it, you need to execute the install_deps.sh script to automatically detect and download the required dependencies. After that, you can run the run.sh script with administrator privileges to automatically open the program, or you can manually execute the Insight_Viewer program in the bin folder.

Figure 3-2 On the Linux side, executed through script

Figure 3-3 On the Linux side, simply run it directly